Legacy SunTrust/Truist Bank Survey Requirements

The following are Truist Bank’s standards for real estate surveys certified to Truist Bank, its successors and/or assigns, as their interest may appear, for the documentation of commercial loans secured by real estate.

- PREPARER

- A land surveyor or professional engineer registered/licensed in the state in which the land is located.

- SURVEYING QUALITY

- Complies with the minimum detail requirements for land title surveys as adopted by the American Land Title Association and American Congress on Surveying and Mapping (ALTA/ACSM).

- Satisfies all title insurer minimum standards for removal of the standard survey exception from the title insurance commitment or binder.

- SURVEY MAP INFORMATION

- Shows the dates of original preparation and of each revision.

- Last dated no earlier than 60 days prior to the date of loan closing.

- Includes the name, address, telephone number, and signature of the surveyor.

- Includes the seal and registration or license number of the surveyor.

- SURVEY ELEMENTS for BOUNDARY SURVEYS

- The direction of magnetic north.

- The boundaries of the property by courses and distances, together with a metes and bounds description corresponding to such survey (which may be written on the survey or on a separate certified page).

- The area of the property in acres and square feet.

- The means of access to the property.

- A beginning point for the property boundary established by a monument located at the beginning point or by reference to a remote monument.

- All lot, section, range lines, and all city, county, and state boundaries shown.

- The street address of the improvements shown.

- All curb cuts, driveways, and fences shown.

- All easements affecting the land shown with the date and recording information for the document creating each easement.

- All structures or improvements on the land shown, including sidewalks and stoops.

- All streets adjacent to the land shown.

- All building and other setback lines shown.

- All encroachments identified.

- All Special Flood Hazard Areas shown.

- A Surveyor's Certificate as described below.

- SURVEY ELEMENTS (IF FOUNDATIONS ARE PRESENT)

- All Boundary Survey elements.

- The location and dimensions of building foundations shown.

- The distance from foundation corners to any adjacent property lines and setback lines.

- SURVEY ELEMENTS (IF PERMANENT STRUCTURES ARE PRESENT)

- All Boundary Survey elements.

- The location and dimensions of buildings, and any other vertical construction.

- The distance from building corners to any adjacent property lines and setback lines.

- All utility lines that service the improvements shown.

- All parking and paved areas shown, together with the number of parking spaces.

- For improvements located within Special Flood Hazard Areas, an identification of the finished ground floor elevations and base flood elevations.

- SURVEYOR'S CERTIFICATE

- Certified to Borrower, Truist (as above), and the title insurer.

- Certification that the survey satisfies all state-required minimum standards.

- Certification that, except as shown on the survey map, no part of the property lies within a Special Flood Hazard Area.

- Certification that, except as shown on the survey map, no encroachments exist.

- Certification that there is pedestrian and vehicular ingress and egress to the property.

- Signed and sealed by the surveyor.

Note:	For loans below $2,500,000, a survey which does not meet the requirement specified above will be acceptable, if based on such survey an acceptable title insurance company (refer to Truist Bank Title Insurance Requirements) commits to issue a title insurance policy without a survey exception and otherwise in compliance with all of the Truist Bank Title Insurance Requirements (including without limitation, as to the inclusion of all endorsements provided for therein).